- Purify Protein Invitro Kinase Assay

- Measure protein expression with absorbance spectrum:

- Set up the configuration, including scale, spectrum, 400-650 should be enough for FRET indicators.

- Clean the asymmetric tube (all crystal).

- Add blank solution, hit Zero, hit baseline.

- Add 1ml indicator solution, hit start, save the data file *.dsw in the correct directory.

- Purify Protein and In vitro kinase assay:

- Transform PRSET plasmids to JM109 (DE3) (from Promega).

- Under ex 480, green gargo, pick up bright colonies.

- put in 2ml 100uM amp LB medium, o/n shake 250rpm 37oC.

- take bacteria medium, put in 100ml 100uM amp LB medium, shake 250rpm, 2-4hr, monitor the OD 600 reading, 0.2-0.4, dilute into (+400ml) 500ml 100uM amp LB medium, add 0.4mM IPTG to induce, shake o/n, RT, with air venturation.

- spin down 7000rpm, 10min.

- add 15 ml bper+0.5 protease coctail tablet+100uM PMSF, completely suspend gently, gentle rock at RT 10min, covered with foil.

- spin 15000rpm 15 min. filter through (optional). Add Ni-NTA agarose beads 1ml , gentle rock at RT 1hr.

- prepare the column, rinse with 2x10ml 1xTNS, 50mM Tris,HClph=7.4, 300mM NaCl.

- Rinse 1x10ml 50mM Tris,HClph=7.4, 300mM NaCl, 10mM imidazole.

- elute with 1x10ml 50mM Tris,HClph=7.4, 300mM NaCl, 100mM imidazole.

- dialysis the protein solution 4oC, 4hr-o/n, in 50 mM Tris pH 8, 100 mM NaCl, 10 mM MgCl2, 2mM DTT. 10x Src kinase buffer stock solution: 500 mM Tris pH 8, 1 M NaCl, 100 mM MgCl2, dilute 10x, add 1:1000 2M DTT.

- Measure the absorbance following the above protocol.

- YFP (extinction coefficient) EC=77000 M-1 CM-1 , CFP EC=32500 M-1 CM-1, GFP EC=62000 M-1 CM-1. So Concentration=reading/EC

- In vitro kinase assay

- Find the 3-way cuvette. Side line facing the emission, small aligned holes for excitation. Clean the cuvette.

- add 100 ul of indicator solution

- put in the holder and cover the cover. Make sure to ajust the aperture to ex=0.5-1 and em=0.5-1

- Hit F10, select HV#950 ON and HV#440 ON.

- Define Exp.

- Emission Scan

- Data acquisition paramter:

  - Number of scan: 1

  - Start:470 End: 530

  - Incremen: 2nm

  - Integration time: 1sec

  - Excitation monochrome: 434

  - Acquisition mode: s / r

- Correct factor file: Mcorrect.spt

  - Measure the peak 526/476, record the number as time goes on, calculate the ratio.

  - Add Src, Add ATP.

- Esp, Esp, Run experiment, Go.

- For Absorbance:

- Scan, connect on line.

- Baseline checked

- Zero for the blank.

- Start for the samples.

- Save data as .csv for ascII format

- For Fluorometer:

- Other functions, output to ascII, F1 to pickup files from directory.

- For Platereader:

- Connect, move in. measurement configuration.

- Dynamics: 3-5 cycles without ATP.

- 60 cycles after ATP, 2 min interval. 30 flashes.

- Increasing step: 5 nm.

- Band width 5nm. Excitation wavelength: 420nm.

- Emission: 471 to 531. temperature: 30oC.