- Alignment of Microscopy

- Align the lights, adjust the knob in the light house to position the light in the center.

- Align the field.

  - switch eyepiece to O.

  - close down the field aperture.

  - Adjust the knobs to center the field in the center and enlarge it to see whether it can fulfill the field.

- Align the phase ring.

  - switch the eyepiece to Boucher, adjust the knob in the eyepiece until see a sharp and clear image.

  - use hand to manually adjust the phase ring until it is centered.

  - Switch the eyepiece to O again, adjust the condenser height.

  - Focus

- Taking Images with time lapse video function:

- Turn on microscopy, slide the filter to direct the light to a correct camera, turn on controllers, monitor

- In the mac computer, go to scion image->special->load macros->time lapse video.txt

- Special->start capture->select regions of interest (alt-A for the whole image)->special->save->select numbers of frames based on calculation->select the time intervals->save